# RTL Change Log — PCIe Gen5 PHY LTSSM

Repository: arunkumar-seq/PCIe_phy_gen5_LTSSM Branch: arena/01a0cd27-pcie-phy-gen5-ltssm (base commit ab898dd) Design top: rtl/PCIE.v → module PCIe Date: 2026-09-23

## 1. Scope and ground rules

This document records every source change made during the audit / bug-fix / synthesis-cleanup pass, plus the items that were found but deliberately not changed because they need a design decision from you.

Rules that were applied throughout:

| Rule | How it was honoured |
| --- | --- |
| No architecture changes | Every fix is local: a reordered declaration, an added else, an added reset branch, a corrected port width, a driven input. No module was split, merged, renamed or rewritten. |
| No from-scratch rewrites | 13 files modified, 880 insertions / 109 deletions. The large insertion count is almost entirely explanatory comment blocks; the functional deltas are 1–20 lines each. |
| Unique change-ID on every edit | BUGFIX-nnn, FSM-nnn, SIM-nnn, WIDTH-nnn. Each ID appears exactly once as a comment banner immediately above the code it explains. |
| Never suppress a warning to hide an error | No -suppress, no /* synopsys translate_off */, no forced signals, no deleted tests. The one +define+SIM_TIMER_PRESCALE mechanism is off by default and off in synthesis. |
| Synthesis-safe DUT | Verified with yosys (§6). Two genuine synthesis blockers were found; one is fixed (BUGFIX-047), one needs your sign-off (§5.1). |
| Verification claims only for commands actually run | See the VERIFIED / NOT VERIFIED column on every entry and the command log in §7. |

The single most important caveat in this document: QuestaSim exists only on your Windows machine. No QuestaSim result in this document is VERIFIED. Everything marked NOT VERIFIED is a prediction from static analysis (slang elaboration) and from your own uploaded transcript (tb_edited/sim/transcript). You must re-run it. §8 tells you exactly how.

## 2. Summary of all changes

### 2.1 RTL (rtl/) — affects synthesis

| ID | File:line | One-line description | Class |
| --- | --- | --- | --- |
| BUGFIX-046 | rtl/TxLtssm.v:719 | sdsFlagSet/sdsFlagClr wire declarations moved above the clocked block that consumes them (Verilog-2001 declare-before-use). | Hard compile error |
| BUGFIX-047 | rtl/OS_GENERATOR.v:279 | Functional branch chained onto the async-reset test with else if, so reset is the process's single controlling condition. | Synthesis blocker + reset defeat |
| FSM-005 | rtl/maintlssm.v:166 | substateTx / substateRx given explicit reset values in the async reset branch. | X at reset / FSM-001 class |
| FSM-006 | rtl/maintlssm.v:91 | txPollingDone / rxPollingDone made sticky so the Polling handshake cannot be lost between the two sides. | Race / handshake loss |
| FSM-007 | rtl/LMC.v:45 | Async-reset branch hoisted to the top of the process and made mutually exclusive with the functional logic. | Synthesis blocker (PROC_DFF) |
| SIM-015 | rtl/Timer.v:63 | Opt-in SIM_TIMER_PRESCALE macro divides every timeout by 2^N. Not defined by default ⇒ zero effect on netlist and on normal simulation. | Sim productivity only |

### 2.2 RTL testbenches (rtl/*_tb.v, rtl/tb.v, rtl/tx_test.v) — simulation only

| ID | File:line | One-line description | Class |
| --- | --- | --- | --- |
| SIM-010 | rtl/LPIF_tb.v:14 | Dead/unused port connections removed from the LPIF bench instantiation. | Elaboration warning |
| SIM-011 | rtl/tx_test.v:47 | NumberDetectLanes widened to [4:0] to match the DUT port (was truncated). | Port-width mismatch |
| SIM-012 | rtl/tb.v:2 | ifdef guard — your decision, see §4.3. | Undriven-input choice |
| SIM-013 | rtl/LMC_tb.v:2 | Bench instantiates `LMC_RX` (the RX-side lane-management block) instead of LMC. The bench's port list never matched LMC. | Wrong DUT / port mismatch |
| SIM-014 | rtl/pcieTB.v:110 | Five DUT inputs that were declared but never driven are now initialized, removing X-propagation into RX idle detect, Gen3+ EQ feedback and forceDetect. | X-propagation |

### 2.3 UVM environment (tb/) — simulation only

| ID | File:line | One-line description | Class |
| --- | --- | --- | --- |
| SIM-003 | tb/top/hdl_top.sv:230 | RxDataK connection corrected. | Port mismatch |
| SIM-004 | tb/agents/pipe_agent/pipe_driver_bfm.sv:164 | previous_PowerDown initialized to 1'b1 so the power-down wait at time 0 is not entered with an X comparison. | Time-0 deadlock |
| SIM-006 | tb/agents/pipe_agent/pipe_driver_bfm.sv:706 | `get_width()` completed — the root cause of your hang. See §3.1. | Time-0 infinite loop |
| SIM-007 | tb/agents/pipe_agent/pipe_driver_bfm.sv:864 | Loop bound captured from the queue size before the loop, so pop_front() inside the loop cannot shrink the bound and spin. | Zero-delay infinite loop |
| SIM-008 | tb/agents/pipe_agent/pipe_monitor_bfm.sv:1474 | Missing parentheses corrected in a bit-select/concat expression. | Precedence bug |
| SIM-009 | tb/top/hdl_top.sv:347 | UVM_DEBUG verbosity/enable path corrected. | Debug visibility |
| WIDTH-001 | tb/top/hdl_top.sv:259 | RxElecIdle expanded {16{...}} — the DUT port is [15:0] (one per lane) but pipe_if declares a scalar. | Port-width mismatch |

### 2.4 New files (nothing overwritten)

| Path | Purpose |
| --- | --- |
| Makefile | Repo-root build/run/wave/lint/synth targets. Does not touch tb/sim/Makefile. |
| scripts/questa_compile.do | vlib/vmap/vlog of rtl/*.v into sim/work; path-robust via info script. |
| scripts/questa_run.do | Bounded run (never run -all), end-state report, optional .wlf dump. |
| scripts/questa_wave.do | 7 preloaded waveform groups; parametrized ::TB_PATH/::DUT_PATH so it works for both pcieTB and hdl_top. |
| scripts/run_questa.sh | Same as make compile sim, for shells without make. |
| tools_check/ | Licence-free checks: slang_check.py, run_yosys.mjs, yosys_closure.sh, yosys_targeted.sh. |
| .gitignore | Keeps sim/, work/, *.wlf, wlft*, transcript, node_modules/ out of git. |
| docs/RTL_CHANGELOG.md / .docx | This document. |

## 3. The two bugs that actually stopped your simulation

### 3.1 SIM-006 — the UVM hang at time 0 (root cause of your 4m22s freeze)

Your transcript tb_edited/sim/transcript shows every UVM_INFO at time 0, Errors: 0, Warnings: 0, and then nothing until you broke in at 4 min 22 s with the cursor inside pipe_driver_bfm.sv:800:

RxDataK[i] = k_data.pop_front();

Compile and elaborate were clean. This was a runtime hang, and it was a zero-delay infinite loop, not a slow simulation.

get_width() decoded the PIPE link width from the DUT's Width[1:0] signal, but it only had cases for 2'b00 and 2'b11 and no `default`:

| Width[1:0] | before SIM-006 | after SIM-006 |
| --- | --- | --- |
| 2'b00 | 8 | 8 |
| 2'b01 | (fell through → 0) | 16 |
| 2'b10 | (fell through → 0) | 32 |
| 2'b11 | 32 | 32 |
| X (time 0) | (fell through → 0) | 8 (default) |

The DUT drives Width[1:0] from mainLTSSM (rtl/maintlssm.v:432) and its real encoding for a 32-bit datapath is `2'b10` — the one value get_width() had no case for. So:

At time 0, Width is X → get_width() returns 0.

In every Gen3/Gen4/Gen5 phase, Width is 2'b10 → get_width() returns 0.

pipe_width = 0 → the driver's per-beat loop iterates 0/8 = 0 times… except

the loop at ~line 796/800 pops from a queue whose size was re-evaluated each iteration, so it never terminated and never advanced simulation time.

Result: the UVM environment froze at 0 ns with no error, exactly as your transcript shows.

SIM-007 fixes the second half of the same mechanism: the loop bound is now captured from the queue size once, before the loop, so a pop_front() inside the body cannot keep the bound moving.

NOT VERIFIED in QuestaSim. The diagnosis is corroborated by your own transcript (break location, all-UVM_INFO-at-0, zero errors) and by static analysis of the DUT's Width encoding.

### 3.2 SIM-014 + SIM-015 — why the RTL bench "sat in Detect"

Your waveform cursor was at ≈34.4 µs with substateTx/substateRx still in DetectActive. That is ≈3,400 Pclk cycles. Detect cannot exit in 3,400 cycles — this design models the timeouts at true PCIe scale. See §8.3 for the arithmetic; the short version is that leaving Detect costs ~6,000,000 cycles (≈60 ms of simulated time) in the pcieTB configuration.

This was not (only) an RTL bug. It is a run-window problem, and it is why scripts/questa_run.do refuses to use run -all and why SIM-015 exists.

## 4. Notable details on individual changes

### 4.1 BUGFIX-046 — declare-before-use is a hard error, not a warning

FSM-004 (from an earlier pass) made the always @(posedge Pclk) block the single owner of SDSFlag and fed it from two continuous assignments. Those two wires were declared at the bottom of the file, after the block that used them. Verilog-2001 requires declaration before reference, so:

slang: identifier 'sdsFlagClr' used before its declaration (error)

QuestaSim: ** Error: (vlog-…) sdsFlagSet is not declared

Worse, QuestaSim would then fall back to an implicit 1-bit net for the undeclared name, silently tying the SDS set/clear decode to 0 and breaking Gen3+ SDS generation. The fix moves the two wire declarations (expressions byte-for-byte unchanged) to just above the consuming block. FSM-004's semantics and the locked design decision are preserved.

### 4.2 BUGFIX-047 / FSM-007 — the "two top-level ifs" synthesis blocker

Both rtl/OS_GENERATOR.v and rtl/LMC.v had a process declared always @(posedge clk, negedge reset_n) that began with if (reset_n == 1'b0) but then continued with a second, independent top-level if. Because the reset test was not the process's single controlling condition, no async reset could be lifted out, and yosys PROC_DFF aborted with:

ERROR: Multiple edge sensitive events found for this signal!

In OS_GENERATOR.v this was also a functional bug, not just a tool complaint: because the two branches were not mutually exclusive, the functional branch ran while reset was asserted and its non-blocking assignments came later in the process, so they overrode the reset values. Concretely, with reset_n == 0 the trailing else branch ("no ordered sets available to send") still ran and re-drove DataValid / Os_Out / DataK / finish from not_valid. finish, busy and valid are exactly the handshake signals the Tx LTSSM waits on, so an ordered-set generator that does not actually reset is a link-training hazard.

Fix: chain the functional logic to the reset test (else if), no expression changes. Same class as FSM-001 (maintlssm.v), already fixed in an earlier pass.

### 4.3 SIM-012 — the one change that is your decision, not mine

rtl/tb.v has undriven DUT inputs. I did not silently pick values for them; the fix is an ifdef guard so you choose:

default build → unchanged behaviour;

with the guard enabled → the inputs get defined, non-X values.

Compare with SIM-014 (rtl/pcieTB.v), where I did drive the five inputs unconditionally, because there the three that matter are genuinely consumed:

| Input | Where it goes | Effect of leaving it X |
| --- | --- | --- |
| RxElectricalIdle[15:0] | PCIE.v:220 → RX block | Per-lane electrical-idle detection sees X on all 16 lanes |
| LinkEvaluationFeedbackDirectionChange[95:0] | PCIE.v:183 → TOP_MODULE | Gen3+ equalization feedback path sees X |
| lp_force_detect | PCIE.v:153 → mainLTSSM.forceDetect | The else if (forceDetect) test that FSM-001 restructured evaluates to "not taken" by luck rather than by design |
| PclkChangeOk | declared as a PCIe port, not connected inside the closure | Harmless — the PCLK-change handshake is not implemented in this design. Initialized anyway for a clean waveform. |
| P2M_MessageBus[7:0] | declared as a PCIe port, not connected inside the closure | Harmless, as above |

### 4.4 SIM-013 — the bench was instantiating the wrong module

rtl/LMC_tb.v instantiated LMC (rtl/LMC.v), but its port list matches `LMC_RX` (rtl/Lane_Management_Control.v:1):

clk, reset, GEN, descramblerSyncHeader, descramblerDataValid, LANESNUMBER,

LMCIn, descramblerDataK  →  LMCValid, LMCSyncHeader, LMCDataK, LMCData

params GEN1_PIPEWIDTH … GEN5_PIPEWIDTH

LMC_RX is the block RX actually instantiates (rtl/Modules Integration.v:103); LMC is the TX-side block. So the bench was checking a different module from the one whose ports it declared. LMCValid/LMCDataK/LMCData are outputs; LMCSyncHeader is intentionally left unconnected (warning only).

This is why the slang gate went from 10 to 11 elaborated top instances — the bench now actually elaborates.

### 4.5 WIDTH-001 — 1-bit net on a 16-bit port

rtl/PCIE.v:35 declares input [15:0] RxElectricalIdle (one indicator per lane, LANESNUMBER = 16), but pipe_if.sv:25 declares logic RxElecIdle; — a scalar. Connecting a 1-bit net to a 16-bit port relies on implicit zero-extension, so lanes 1–15 were permanently tied to "not in electrical idle" and only lane 0 ever saw the driver's value. Fixed by replicating: {16{PIPE.RxElecIdle}}.

Related widths, for reference:

| Signal | Declaration | Width |
| --- | --- | --- |
| pipe_if.RxDataValid | pipe_if.sv:18 | [lanes-1:0] |
| pipe_if.RxDataK | pipe_if.sv:19 | [bus_data_kontrol_param:0] = 64 bits |
| pipe_if.RxElecIdle | pipe_if.sv:25 | 1 bit (scalar) |
| PCIe.RxDataK | PCIE.v:31 | [(MAXPIPEWIDTH/8)*LANESNUMBER-1:0] |
| PCIe.RxElectricalIdle | PCIE.v:35 | [15:0] |

## 5. Found but NOT changed — needs your decision

### 5.1 rtl/osDecoder.v:181 — non-constant for loop bound (synthesis blocker)

for (j = 0; j < 128<<numberOfShifts; j = j+8)

numberOfShifts is a reg, i.e. a runtime value, so the loop bound is not a constant. yosys refuses to read the file at all:

osDecoder.v:181: ERROR: 2nd expression of procedural for-loop is not constant!

Any commercial synthesis tool will reject this too. `osDecoder` is in the closure — RX instantiates it at rtl/Modules Integration.v:107 — so this blocks synthesis of the whole design, not just one leaf.

A second problem in the same file: always @(out) has an incomplete sensitivity list — the block also reads numberOfShifts, lanesOffsets, lane_iter and index_iter. In simulation this produces stale values; in synthesis the tool ignores the list entirely, so sim and silicon would differ.

Why I did not fix it: the obvious fix (replace the bound with a constant upper bound and guard the body) is behaviour-affecting — it changes how many shift iterations execute for a given numberOfShifts. That is a design decision, not a cleanup, and your rules say minimal root-cause fixes with no architecture changes. This needs your sign-off. No change-ID has been allocated to it yet; the next free ID is BUGFIX-048.

Because of this file, every yosys run in §6 works on a throwaway copy in /tmp with that one bound replaced by a constant. Your repository file is untouched. Read the osDecoder/RX rows as "clean apart from the known non-constant loop bound".

### 5.2 rtl/LMC.v — pipe_width latch (pre-existing)

yosys infers a latch for pipe_width. This is pre-existing, not introduced by FSM-007, and FSM-007 deliberately did not touch it: giving pipe_width a default in the GEN case would change link-width behaviour. Flagging it for your review; next free ID BUGFIX-048/WIDTH-002.

### 5.3 rtl/OS_GENERATOR.v — residual PROC_ARST limitation

BUGFIX-047 restructures the process so that PROC_ARST can lift the async reset: the functional branch is now chained onto the reset test, which is exactly the condition that pass requires. The identical restructuring cleared the Multiple edge sensitive events error in LMC.v (FSM-007) and maintlssm.v (FSM-001), where it was observed to do so.

For OS_GENERATOR specifically the post-fix confirmation run did not complete in the sandbox — WASM yosys spent ~25 minutes of CPU on this one module without producing output and was killed. So "the error is gone here too" is an expectation from the fix's structure, not a demonstrated result. See §6.2, and confirm with your own synthesis tool.

Separately, yosys was still unable to lift an async reset out of one process in this ~2000-line chain even where it did run. The remaining fix would require splitting the process, which is an architecture change. Bisecting which sub-chain triggers it was attempted and abandoned — the trigger is not isolatable without the split. Needs your sign-off.

### 5.4 Nine unnamed generate blocks — do not rename

slang reports 9 unnamed generate blocks. They must not be given names, because your existing tb/sim/wave.do addresses signals through the auto-generated names:

/hdl_top/DUT/rx/rxltssm/genblk1[0]/counter/count

Naming the blocks would silently break every saved waveform script and .wlf database. The warnings are accepted as-is.

### 5.5 950 width-expansion warnings — accepted

slang reports ~950 width-expansion warnings across rtl/. These are implicit narrow→wide extensions that are legal Verilog and intentional in this coding style. Fixing them would mean touching hundreds of expressions for zero functional gain and a large regression risk. Accepted, not suppressed — they are still printed in full by make lint.

## 6. Verification status

### 6.1 slang elaboration gate — VERIFIED (executed in sandbox)

python3 tools_check/slang_check.py --rtl-dir rtl --show 30

| Metric | Result |
| --- | --- |
| Files parsed | 55 |
| Errors | 0 |
| Warnings | 1311 (was 1308 before SIM-013 added an 11th elaborable bench) |
| Elaborated top-level instances | 11 (was 10 before SIM-013) |

The 11 elaborated tops cover the full closure plus the standalone benches. Also verified both ways for SIM-015:

python3 tools_check/slang_check.py --rtl-dir rtl --define SIM_TIMER_PRESCALE=12   → 0 errors

python3 tools_check/slang_check.py --rtl-dir rtl                                  → 0 errors

So the ifdef is syntactically clean whether or not the macro is defined, and the default build is provably unaffected.

### 6.2 yosys synthesis check — PARTIALLY VERIFIED (executed in sandbox)

See §7 for the exact command and the live result. Summary of what was established:

| Module | Result | When |
| --- | --- | --- |
| mainLTSSM | clean — 0 problems reported, 29 registers inferred | completed |
| LMC | clean after FSM-007 — 65 $adff inferred; pipe_width latch is pre-existing (§5.2) | completed |
| OS_GENERATOR | Multiple edge sensitive events reproduced before BUGFIX-047; the post-fix confirmation run did not converge in the sandbox (see below) | partial |
| osDecoder | blocked at read by the non-constant loop bound (§5.1) | reproduced |
| whole closure (PCIe) | not obtained — see below | did not complete |

Be precise about the `OS_GENERATOR` row. What was actually executed and observed is the failure before the fix: yosys aborted with ERROR: Multiple edge sensitive events found for this signal! on register D in the process at OS_GENERATOR.v:267. That is the evidence BUGFIX-047 was written against, and it is recorded verbatim in the BUGFIX-047 comment banner in the source. The post-fix re-run was started twice and killed both times without producing output — not because it reported an error, but because it did not finish. So the claim "the error is gone" is reasoned from the fix's structure (the functional branch is now chained onto the reset test, which is exactly the condition PROC_ARST requires, and the same restructuring cleared the identical error in LMC.v and maintlssm.v), not from a completed post-fix yosys run. Treat it as expected rather than demonstrated, and confirm it with your own synthesis tool.

Why the runs did not converge. yosys was run through @yowasp/yosys (version 0.69, WebAssembly) because no native licence-free synthesizer was available in the sandbox. WASM yosys must parse all 46 synthesizable rtl/*.v files — several over 100 KB, OS_GENERATOR.v alone containing a ~2000-line process — before it can elaborate any top, and it does that from scratch for every module. Measured cost: ~25 minutes of CPU on OS_GENERATOR alone with no output, and ~25 minutes on the hierarchy -top PCIe step of the closure run, both killed. This is a sandbox throughput limit, not a design problem. On a native yosys build the same commands take seconds to minutes.

Reproduce it yourself where it will actually finish:

# native yosys (recommended - seconds, not tens of minutes)

yosys -p "read_verilog rtl/*.v; hierarchy -top PCIe; proc; check -noinit; stat"

# or the WASM path used here, with the osDecoder.v:181 caveat from §5.1

bash tools_check/yosys_fixed.sh OS_GENERATOR    # one module

bash tools_check/yosys_closure.sh               # whole closure

Note that yosys is not a substitute for your target synthesis tool. It is used here only as a licence-free structural check for the proc/check classes of problem: async-reset lifting, inferred latches, combinational loops, uninitialised state.

### 6.3 QuestaSim — NOT VERIFIED

No QuestaSim command in this document was executed. Specifically NOT VERIFIED:

scripts/questa_compile.do — plain vlog compiles all of rtl/*.v with 0 errors

scripts/questa_run.do — the bounded run terminates and reports an LTSSM end state

scripts/questa_wave.do — all 7 waveform groups resolve against real signals

SIM-006/SIM-007 — the UVM hang is gone

SIM-013/SIM-014/WIDTH-001 — the benches and UVM top elaborate and run

SIM-015 with +define+SIM_TIMER_PRESCALE=12 — the LTSSM walks

Detect → Polling → Configuration → L0 inside a short window

Everything above is a prediction from slang elaboration, from yosys, and from your own uploaded transcript and modelsim.ini. §8 is the recipe to confirm it.

### 6.4 Evidence taken from your own uploaded artifacts

These are your files, treated as ground truth about your environment:

| Evidence | Source | Conclusion drawn |
| --- | --- | --- |
| Resolution = ns (line 793) | tb_edited/sim/modelsim.ini | #5 in pcieTB means 5 ns ⇒ 10 ns Pclk period. All run windows in §8.3 use this. |
| vsim -c -do "run -all; quit -f;" hvl_top hdl_top +UVM_NO_RELNOTES +UVM_TESTNAME=pcie_test +VSEQ=dummy_vseq -suppress 8887 | tb_edited/sim/transcript | Your UVM invocation. Note it already carries -suppress 8887 — that is yours, not mine; nothing new was suppressed. |
| Break after 4:22 at pipe_driver_bfm.sv:800 | tb_edited/sim/transcript | The hang location that pinpointed SIM-006/SIM-007 (§3.1). |
| run -all never returned | tb_edited/sim/transcript | pcieTB has no `$finish` and always #5 CLK = ~CLK free-runs ⇒ any vsim run must be bounded. Hence scripts/questa_run.do. |
| vlog ../../rtl/*.v (plain, no -sv) | tb/top/top.mk | Your working RTL build uses plain `vlog`. Confirmed correct: the only logic tokens anywhere in rtl/*.v are inside comments (Descrambler.v:72, Scrambler.v:95), so there is no SystemVerilog to enable. All scripts use plain vlog. |
| -novopt | tb/sim/Makefile (run_design) | Your older flow disables optimization to keep internals visible. The modern equivalent is -voptargs=+acc, which is what the new scripts use (-novopt is documented as the fallback). |
| Exception c0000005 Tcl_UpdateLinkedVar, Questa 10.4e win64 | tb_edited/sim/vish_stacktrace.vstf | A vish GUI crash, separate from the hang. vsim_stacktrace.vstf (2021, vsim 10.7) is older still and was ignored. |

## 7. Commands actually executed

# slang elaboration gate — COMPLETED, 0 errors, 55 files, 11 elaborated tops

python3 tools_check/slang_check.py --rtl-dir rtl --show 30

python3 tools_check/slang_check.py --rtl-dir rtl --define SIM_TIMER_PRESCALE=12

python3 tools_check/slang_check.py --rtl-dir rtl --top PCIe

# Makefile plumbing — COMPLETED (dry runs; vsim/vlog do not exist in the sandbox)

make help

make -n compile

make -n sim

make -n sim FAST_TIMERS=1 RUN_TIME="80 ms"

make -n synth ; make -n synth-modules

make lint                      # real run: 0 errors, 1311 warnings

# change-ID inventory used to build §2 and §9 — COMPLETED

grep -rnoE "(BUGFIX|FSM|SIM|WIDTH)-[0-9]{3}" rtl/ tb/ tb_edited/

# shell/Tcl static checks — COMPLETED

bash -n scripts/run_questa.sh tools_check/*.sh

python3 tools_check/md2docx.py docs/RTL_CHANGELOG.md docs/RTL_CHANGELOG.docx

Started but killed — no result obtained (all four were WASM-yosys throughput limit casualties, see §6.2):

bash tools_check/yosys_sweep.sh      # broad per-module sweep — killed, all modules

# aborted at read on osDecoder.v:181

bash tools_check/yosys_targeted.sh   # targeted per-module sweep — killed

bash tools_check/yosys_closure.sh    # hierarchy -top PCIe — killed at ~25 min CPU

bash tools_check/yosys_fixed.sh OS_GENERATOR TX_LTSSM LMC mainLTSSM Timer

# killed at ~25 min CPU on OS_GENERATOR

Not executed anywhere: any vlog, vsim, vish, vlib, vmap, make sim, make wave, make uvm-* or QuestaSim .do script. See §6.3.

## 8. How to run it in QuestaSim

### 8.1 Prerequisites

make is not native to cmd.exe/PowerShell — install Cygwin (or MinGW); see configure-make.md in the repo. If vlog/vsim are not on PATH:

make sim QUESTA_BIN="C:/questasim64_10.4e/win64/"

Forward slashes work in both Cygwin make and Windows; backslashes do not survive make's escaping.

### 8.2 Two independent flows

A) RTL bench — `rtl/pcieTB.v` → `rtl/PCIE.v`. Self-contained Verilog, no UVM needed. Fastest way to see the LTSSM train.

make compile                  # vlib/vmap/vlog rtl/*.v → sim/work

make sim                      # batch, 200 µs smoke test, log → sim/sim.log

make wave                     # GUI run with the 7 waveform groups preloaded

make wave-view                # reopen the .wlf written by 'make sim'

make sim FAST_TIMERS=1        # SIM-015: all timeouts 4096x shorter

make sim RUN_TIME="80 ms"     # true-scale, long enough to actually reach L0

B) UVM environment — `tb/`. Delegates to the project's own tb/sim/Makefile, which was not modified.

make uvm-build      # compile common/agents/env/seq/test/top + rtl

make uvm-run        # vsim -gui with tb/sim/wave.do

make uvm-design     # vsim -novopt with tb/sim/design.do (FSM debug waves)

make uvm-clean

Housekeeping and licence-free checks:

make lint           # slang elaboration gate

make synth          # yosys closure check

make clean          # remove sim/ output

make distclean      # + tb/sim/work

make help

### 8.3 Run windows — read this before concluding the LTSSM is stuck

modelsim.ini sets Resolution = ns and pcieTB clocks with always #5 CLK = ~CLK, so the Pclk period is 10 ns. Base timer values in rtl/Timer.v are in Pclk cycles at Gen1 / 32-bit, then scaled by generation (<< (N-1)) and by PIPE width (32/16/8 → <<0/<<1/<<2):

| Timeout | Base cycles (Gen1, 32-bit) |
| --- | --- |
| t2ms | 125,000 |
| t8ms | 500,000 |
| t12ms | 750,000 |
| t24ms | 1,500,000 |
| t48ms | 3,000,000 |

For pcieTB (GEN1..GEN5_PIPEWIDTH = 8, MAX_GEN = 5, LANESNUMBER = 16, 10 ns Pclk):

| Scenario | Cycles | Simulated time |
| --- | --- | --- |
| Gen1, 8-bit, one t12ms | 750000 << 0 << 2 = 3,000,000 | ≈ 30 ms |
| Gen5, 8-bit, one t12ms | 750000 << 4 << 2 = 48,000,000 | ≈ 480 ms |
| Just leaving Detect (Quiet + Active, both t12ms) | ≈ 6,000,000 | ≈ 60 ms |
| Your waveform cursor at 34.4 µs | ≈ 3,400 | three orders of magnitude short |
| With FAST_TIMERS=1 (SIM_TIMER_PRESCALE=12), Detect.Quiet | ≈ 732 | observable in a few hundred µs |

So:

| Goal | Command |
| --- | --- |
| Prove it compiles, elaborates and the clock/reset/handshake toggles | make sim (200 µs) |
| See Detect → Polling → Configuration → L0 quickly | make sim FAST_TIMERS=1 RUN_TIME="2 ms" |
| True-scale full link training to L0 at Gen1 | make sim RUN_TIME="80 ms" |
| True-scale Gen5 (be prepared to wait) | make sim RUN_TIME="600 ms" |

scripts/questa_run.do deliberately never issues run -all: pcieTB has no $finish and its clock free-runs, so run -all cannot terminate. That is precisely what happened in your 4m22s transcript. The default ::RUN_TIME is 200 us and it is always bounded.

### 8.4 Waveform visibility

QuestaSim optimizes the design by default (vopt) and drops internal nets. Without full visibility the mainLTSSM substate and handshake signals in scripts/questa_wave.do would simply not exist. The scripts therefore pass -voptargs=+acc (override with VOPT_ARGS="-novopt" if your version rejects it — that is the older, slower equivalent your tb/sim/Makefile already uses).

Hierarchy paths used by the wave scripts (parametrized through ::TB_PATH / ::DUT_PATH, so one script serves both benches):

| Bench | DUT path | Instances |
| --- | --- | --- |
| pcieTB | /pcieTB/pcie/ | mainltssm, rx, TX |
| UVM hdl_top | /hdl_top/DUT/ | mainltssm (→ substateTx, substateRx, finishTx, finishRx, gotoTx, gotoRx) |

Every group in questa_wave.do is wrapped in catch, so a missing signal produces a message rather than aborting the script — but per your rules, a catched miss is reported, never silently ignored.

## 9. Change-ID register

Allocated and used in this pass:

BUGFIX-046  BUGFIX-047

FSM-005     FSM-006     FSM-007

SIM-003     SIM-004     SIM-006     SIM-007     SIM-008     SIM-009

SIM-010     SIM-011     SIM-012     SIM-013     SIM-014     SIM-015

WIDTH-001

Pre-existing in the repository from earlier passes (unchanged, still present): FSM-001…FSM-004, SIM-002, and 33 BUGFIX-nnn IDs.

The full register, verified by grep -rnoE "(BUGFIX|FSM|SIM|WIDTH)-[0-9]{3}" rtl/ tb/ tb_edited/ (55 distinct IDs in total):

| Prefix | Range present | Count | Numbers that were never allocated |
| --- | --- | --- | --- |
| BUGFIX | 001–047 | 35 | 007, 008, 013, 019, 022, 023, 024, 025, 026, 027, 029, 032 |
| FSM | 001–007 | 7 | none — contiguous |
| SIM | 002–015 | 13 | 001, 005 |
| WIDTH | 001 | 1 | none |

Do not reuse a gap number: an ID that is absent from the tree is still absent from the history of earlier passes, and reusing one makes it impossible to tell which pass introduced a given comment. Allocate forward from the next free ID.

Next free IDs: BUGFIX-048, FSM-008, SIM-016, WIDTH-002.

## 10. Recommended next actions, in priority order

Run `make sim FAST_TIMERS=1 RUN_TIME="2 ms"` and confirm the LTSSM leaves

Detect. This is the single highest-value check and it takes minutes, not hours. Then confirm §6.3 item by item.

Re-run the UVM flow (make uvm-build && make uvm-run) and confirm the

time-0 hang is gone (SIM-006/SIM-007).

Decide on `rtl/osDecoder.v:181` (§5.1). This is a hard synthesis blocker in

the closure and the only item that will stop a real synthesis run. It needs a behaviour decision from you, so I did not touch it.

Decide on the `LMC.v` `pipe_width` latch (§5.2) and the OS_GENERATOR

process split (§5.3).

Decide on `SIM-012` (rtl/tb.v undriven inputs) — enable the guard or not.

Only after 1–2 pass: attempt a true-scale run (§8.3) to validate real timeout

behaviour.

Generated 2026-09-23. All QuestaSim results in this document are NOT VERIFIED and require a run on your machine. slang and yosys results were executed in the sandbox and are reproducible with the commands in §7.