# Up/Down Counter
RTL + Complete UVM

A beginner-friendly code walkthrough and Windows QuestaSim lab guide

Audience: a fresher in Design Verification who knows basic digital logic and is starting SystemVerilog/UVM.

Edition: 27 October 2026 • Default counter width: 8 bits • UVM 1.2 recommended

# What you receive

Synthesizable RTL and a layered, reusable UVM testbench organized by responsibility.

Windows batch launchers, a native Questa Tcl compile/run flow, and directed/random tests.

A reference-model scoreboard, functional coverage and an independent RTL-only smoke test.

The full source listing in this document, with per-file explanations before each listing.

Validation boundary: static compilation and elaboration have been checked. Windows Questa simulation has NOT been executed in the development sandbox. No pass waveform, runtime coverage percentage or simulator-version certification is claimed.

# How to read this guide

Read Sections 1–4 first, run a wrap test using Section 5, then study the source walkthrough in Section 9 while opening the matching files. The listings are complete snapshots; the files in the ZIP are the editable source of truth. Use Word’s Navigation Pane to jump between headings.

# 1. The problem: what are we verifying?

A counter is a register that changes by one on a clock edge. An up/down counter can either increment or decrement. RTL describes the intended hardware; a verification testbench stimulates the hardware and checks whether it behaves according to a separate specification.

| Control / output | Meaning |
| --- | --- |
| clk | DUT updates on the rising edge. |
| rst_n | Active-low asynchronous reset: zero means reset, including between clock edges. |
| clear | At the next rising edge, set count to zero. |
| load / load_value | At the next rising edge, copy load_value into count. |
| enable / up | If enabled, up=1 increments and up=0 decrements; otherwise hold. |
| count | Unsigned WIDTH-bit state, observed by the monitor. |

Priority is reset > clear > load > enabled count > hold. Priority matters when multiple controls are asserted together. For example, clear=1 and load=1 must produce zero, not load_value. Load must still work when enable=0.

At WIDTH=8, values range from 0 to 255. Incrementing 255 produces 0; decrementing 0 produces 255. This is modulo-256 arithmetic, not saturation. RTL permits positive WIDTH; the UVM constraints and coverage assume WIDTH >= 2.

# 2. SystemVerilog and UVM vocabulary

| Term | Beginner explanation |
| --- | --- |
| module | Static hardware structure elaborated by the simulator. The DUT and tb_top are modules. |
| interface | A shared bundle of signals plus timing rules. Avoids passing every pin separately to each class. |
| class / object | A class is a software-like blueprint. new or factory create makes an object at runtime. |
| component | A class with a name, parent and UVM phases. Components form the testbench hierarchy. |
| transaction / sequence item | A data object describing one operation; here, controls for one active clock edge. |
| sequence | A task-based generator of transaction objects. It is not a component. |
| sequencer | Manages sequence-to-driver handshakes and arbitration. |
| virtual interface | A class-held reference to an already elaborated interface instance; not another copy of the pins. |
| factory | Creates registered objects/components by type; enables later substitution with overrides. |
| analysis port | A broadcast connection used by the monitor to publish observed transactions. |
| scoreboard | An independent reference model plus actual-versus-expected comparison. |
| coverage | A measure of which planned situations have been observed; not proof of correctness. |

# 3. Two hierarchies, one connected system

## 3.1 Static HDL hierarchy

tb_top

clk : generated clock

vif : counter_if #(WIDTH)

dut : up_down_counter #(WIDTH)

## 3.2 Runtime UVM hierarchy

uvm_root

uvm_test_top : counter_base_test or a derived focused test

env : counter_env

agent : counter_agent

sequencer : counter_sequencer   (active mode only)

driver    : counter_driver      (active mode only)

monitor   : counter_monitor

scoreboard : counter_scoreboard

coverage   : counter_coverage

A source folder is not a runtime instance. The package is a namespace, not hardware. Sequence and item objects do not appear as child components in the UVM tree. The static interface is passed by reference to driver and monitor using uvm_config_db.

## 3.3 The transaction journey

Test starts sequence

-> sequence creates counter_item and fills its fields

-> sequencer gives the item to the driver

-> driver applies controls through the interface

-> DUT updates count on a rising edge

-> monitor samples actual pins and makes a NEW item

-> analysis port broadcasts it to scoreboard and coverage

The scoreboard never trusts an expected result computed by the driver. It predicts count using the observed controls. This separation is important: a driver bug should not silently dictate the expected output too.

# 4. Timing, reset and the reference model

## 4.1 One item means one active clock edge

The clock period is 10 ns. The driver waits for the falling edge and uses a clocking block with output #0 to apply controls there. This gives the DUT half a cycle before its next rising edge. The DUT schedules count updates with nonblocking assignment (<=). The monitor clocking block uses input #0 at the rising edge, which samples in the Observed region after DUT NBA updates. Thus the monitor sees the result of the controls for that same edge.

Do not replace monitor input #0 with #1step without also redesigning the scoreboard timing: #1step observes the value just before the edge and therefore the previous count. The chosen #0 is intentional here, not a generic recommendation for every testbench.

| Example item | Controls | Expected count after rising edge |
| --- | --- | --- |
| Reset | rst_n=0 | 0 |
| Load maximum | rst_n=1, load=1, value=255 | 255 |
| Increment | load=0, enable=1, up=1 | 0 |
| Decrement | enable=1, up=0 | 255 |
| Hold | enable=0 | 255 |
| Clear and load | clear=1, load=1, value=123 | 0 |

## 4.2 Predicting and comparing

For each observed valid cycle, the scoreboard applies reset/clear/load/count/hold priority to its private expected variable. WIDTH-bit assignment naturally truncates arithmetic at the boundary. It then compares tr.count against expected using !==. Case inequality reports unknown (X) or high-impedance (Z) count as a mismatch instead of hiding it.

initialized prevents comparing an uninitialized reference state. All supplied sequences begin with reset. checked counts comparisons and mismatches counts incorrect responses; check_phase flags a run with no checked traffic. report_phase prints a summary.

## 4.3 Drain cycles and asynchronous reset scope

valid is a testbench-only signal, not a DUT input. Before the first item it is zero; the driver sets it to one and keeps driving the final controls while idle. Consequently monitor checks continue during the two-cycle drain, so checked samples may exceed the requested item count. This is expected.

The UVM sequences assert asynchronous reset on a falling edge and check the resulting zero at a rising edge. They test reset dominance but do not independently prove the before-next-edge asynchronous behavior. The separate RTL smoke test asserts reset between rising edges and checks before the next one. A production environment could add dedicated asynchronous-reset assertions and randomized sub-cycle reset pulses.

# 5. Windows QuestaSim: first successful run

Requirements: a licensed UVM-capable QuestaSim/ModelSim edition supporting classes, constraints and covergroups. Limited/free editions may lack these features. The ZIP contains no simulator, UVM installation or license. Use your local UVM source tree with src/uvm_pkg.sv; compiling it locally avoids vendor-specific precompiled-library mappings.

## 5.1 Command Prompt batch flow

Extract the ZIP fully. Do not run from inside the ZIP preview. Replace the installation paths below with your real directories. The commands use cmd.exe syntax, not PowerShell syntax.

cd /d C:\DV\up_down_counter

set "PATH=C:\questasim64_2024.3\win64;%PATH%"

set "UVM_HOME=C:\questasim64_2024.3\verilog_src\uvm-1.2"

run.bat

run.bat counter_wrap_test 1 8 1000

run.bat counter_random_test 42 16 5000

regression.bat

run.bat arguments are test name, seed, width and random-item count. Defaults are counter_base_test, 1, 8 and 1000. regression.bat runs the five named tests and stops at the first nonzero return code. It recompiles per test for simplicity; the optional Makefile regression compiles once.

## 5.2 GUI with waveforms

Open QuestaSim, then enter this in its Transcript. Tcl braces protect paths containing spaces; forward slashes avoid backslash escape problems.

cd {C:/DV/up_down_counter}

set UVM_HOME {C:/questasim64_2024.3/verilog_src/uvm-1.2}

set GUI_MODE 1

set TEST counter_wrap_test

set WIDTH 8

set SEED 1

set NUM_ITEMS 1000

do run.do

The script compiles, loads tb_top with visibility enabled, adds interface signals to Wave and runs to completion. Select Zoom Full to see the entire trace. Inspect clk, rst_n, load, enable, up, load_value and count. GUI mode leaves the simulator open. Inspect /tb_top/test_passed: 1 means no UVM errors/fatals. To run another test, enter quit -sim, change TEST, then do run.do again. Explicitly change WIDTH and NUM_ITEMS if desired because Tcl variables persist.

## 5.3 Compile order and output files

Compile order is UVM package -> RTL -> interface -> counter_pkg -> tb_top. The package includes the .svh class files in dependency order; do not compile those headers independently. The RTL smoke top is not part of the UVM compilation. Source compilation uses UVM_NO_DPI, so no external UVM DPI build is required.

Generated work libraries and logs go under sim/build. A typical log is counter_wrap_test_seed1.log. Static configuration, compile or load failures stop batch execution. After UVM finishes, tb_top sets test_passed from report-server severity counts. sim/run.do examines that flag and exits with code 0 or 1. The 10 ms simulation-time watchdog catches stuck objections or handshakes. A GUI stop is not automatically a pass: inspect the summary.

# 6. UVM phases and common methods

| Method / phase | What this environment does |
| --- | --- |
| new | Calls the superclass constructor; creates analysis ports/exports where needed. |
| build_phase | Creates child components via the factory and obtains virtual interfaces from config DB. |
| connect_phase | Connects sequencer to driver and monitor to scoreboard/coverage. |
| run_phase | Executes timed behavior: sequences, driving and monitoring. Driver/monitor loops run concurrently. |
| raise_objection / drop_objection | Keeps the run phase alive while the test sends traffic and drains monitor callbacks. |
| check_phase | Checks that comparisons actually occurred. |
| report_phase | Prints checked/mismatch counts and coverage percentage. |

Build and connect are functions and must not consume simulation time. run_phase, sequence body and driver operations are tasks and may wait for clocks. A forever loop in a driver is normal: UVM terminates phase tasks after objections drop. Without objections the simulation may end before meaningful traffic occurs.

# 7. Test plan, randomization and coverage

| Test class | Stimulus and purpose |
| --- | --- |
| counter_reset_test | Startup reset, reload, mid-traffic reset; reset overrides all other controls. |
| counter_directed_test | Count up and down, disabled hold in both directions, load, clear priority and immediate reversals. |
| counter_wrap_test | Load maximum; count up to zero and down to maximum. |
| counter_random_test | Reset then weighted randomized items; NUM_ITEMS selects the count after reset. |
| counter_base_test | Default: runs all four sequences serially in one simulation. |

Random constraints make reset deasserted most of the time, clear relatively rare, load occasional and enable usually high. Boundary-biased load_value makes zero and maximum more likely than a uniform distribution would. dist weights describe relative selection weights, not a promise of exact percentages for a finite seed. :/ distributes a total weight across a range, whereas := weights individual values.

The seed makes a failing run reproducible within the same simulator/tool setup and source version. Reproduce the original test, seed, width and item count together. Different simulators or versions can have different random streams.

Coverage records effective operation, output zero/maximum/interior, up/down wrap events and direction×enable, clear×load and reset×load combinations. The wrap calculation uses previous observed count and current effective operation. Coverage does not replace correctness checks: a wrong output can still hit a bin. Review coverage together with a clean scoreboard.

There is no mandatory coverage percentage threshold or code-coverage/UCDB merge flow in this example. A single seed is not coverage closure. Unhit bins should motivate additional directed tests or seeds, not be ignored simply because the scoreboard passed.

# 8. Debugging checklist and learning exercises

| Symptom | Likely cause / next step |
| --- | --- |
| vsim not found | Add your actual Questa win64 directory to PATH. Confirm where vsim finds it. |
| UVM_HOME error | Point to the source root containing src/uvm_pkg.sv, not directly to src or to a compiled library. |
| License / feature failure | Verify your edition/license supports SystemVerilog classes, randomization and covergroups. |
| Unknown counter_* class | Do not compile headers separately. Use root run.do and check the package include order. |
| NOVIF fatal | Check virtual interface type/width and the config DB path uvm_test_top*. |
| MISMATCH | Read the first failure, reproduce the seed, inspect controls/count around that sample and compare priority rules. |
| Count appears one cycle late | Check sampling region and whether you changed clocking block input skew. |
| No traffic or watchdog | Check objections, sequence start path and get_next_item/item_done handshake. |
| Low functional coverage | Run the base test and more seeds; inspect what is unhit before adding targeted stimulus. |
| GUI ended but success unclear | Review UVM_ERROR/UVM_FATAL totals and examine /tb_top/test_passed. |

Before looking at a waveform, calculate the expected wrap-test counts on paper.

Change the load priority in a local copy of the RTL and verify a directed test detects the bug. Restore the correct RTL afterward.

Add a sequence that loads each boundary and holds enable low for several cycles.

Add a coverpoint for consecutive direction reversals, then write a sequence that hits it.

Explain why a monitor must create a fresh observed item instead of publishing the driver’s request handle.

Extend the design with a terminal-count output only after updating the specification, interface, item, monitor and scoreboard together.

# 9. Complete file-by-file code walkthrough

Each following subsection identifies the role and important lines, then lists the entire corresponding file. Code lines are numbered for discussion only; the downloadable source files do not contain these number prefixes.

## 9.1  rtl/up_down_counter.sv

WIDTH sets the register and input-vector size. count is logic because an always_ff block assigns it.

always_ff triggers at a positive clock edge or a negative reset edge. The if/else chain expresses priority. There is intentionally no final else assignment: in a clocked block the register retains its value.

The +/− one expressions are assigned back to a WIDTH-bit variable, which implements modulo wraparound. Nonblocking assignments model synchronous state updates.

001  `timescale 1ns/1ps

002  module up_down_counter #(

003    parameter int unsigned WIDTH = 8

004  ) (

005    input  logic                 clk,

006    input  logic                 rst_n,

007    input  logic                 clear,

008    input  logic                 load,

009    input  logic                 enable,

010    input  logic                 up,

011    input  logic [WIDTH-1:0]     load_value,

012    output logic [WIDTH-1:0]     count

013  );

014    // Priority: asynchronous reset > synchronous clear > load > count > hold.

015    // Assignment truncation implements modulo 2**WIDTH arithmetic.

016    always_ff @(posedge clk or negedge rst_n) begin

017      if (!rst_n)       count <= '0;

018      else if (clear)   count <= '0;

019      else if (load)    count <= load_value;

020      else if (enable) begin

021        if (up) count <= count + 1'b1;

022        else    count <= count - 1'b1;

023      end

024    end

025  endmodule

## 9.2  tb/top/counter_if.sv

The interface owns the signals shared by DUT, driver and monitor. Initial values provide deterministic startup; valid is verification-only.

drv_cb drives at negedge clk with output #0. mon_cb samples at posedge clk with input #0 after NBA updates. The opposite clock edges avoid a DUT/driver race.

001  `timescale 1ns/1ps

002  interface counter_if #(parameter int WIDTH = 8) (input logic clk);

003    logic rst_n = 1'b0;

004    logic clear = 0, load = 0, enable = 0, up = 1;

005    logic [WIDTH-1:0] load_value = '0;

006    logic [WIDTH-1:0] count;

007    logic valid = 0; // Testbench-only marker; not a DUT port.

008

009    // Drive half a cycle ahead of the active DUT edge.

010    clocking drv_cb @(negedge clk);

011      default output #0;

012      output rst_n, clear, load, enable, up, load_value, valid;

013    endclocking

014    // #0 samples in the observed region, after DUT nonblocking updates.

015    clocking mon_cb @(posedge clk);

016      default input #0;

017      input rst_n, clear, load, enable, up, load_value, count, valid;

018    endclocking

019  endinterface

## 9.3  tb/agent/counter_item.svh

rand bit fields represent stimulus; count is a four-state logic response so X/Z output is preserved. Constraints affect randomize(), not directed assignments.

uvm_object_utils_begin/end register the item with the factory and field automation. The constructor forwards the instance name to uvm_sequence_item.

001  class counter_item extends uvm_sequence_item;

002    rand bit rst_n, clear, load, enable, up;

003    rand bit [WIDTH-1:0] load_value;

004    logic [WIDTH-1:0] count;

005

006    constraint typical_traffic {

007      rst_n dist {1 := 98, 0 := 2};

008      clear dist {0 := 95, 1 := 5};

009      load dist {0 := 85, 1 := 15};

010      enable dist {1 := 80, 0 := 20};

011      load_value dist {'0 := 10, MAX_COUNT := 10, [1:MAX_COUNT-1] :/ 80};

012    }

013    `uvm_object_utils_begin(counter_item)

014      `uvm_field_int(rst_n, UVM_DEFAULT)

015      `uvm_field_int(clear, UVM_DEFAULT)

016      `uvm_field_int(load, UVM_DEFAULT)

017      `uvm_field_int(enable, UVM_DEFAULT)

018      `uvm_field_int(up, UVM_DEFAULT)

019      `uvm_field_int(load_value, UVM_DEFAULT)

020      `uvm_field_int(count, UVM_DEFAULT)

021    `uvm_object_utils_end

022    function new(string name = "counter_item"); super.new(name); endfunction

023  endclass

## 9.4  tb/agent/counter_sequencer.svh

This typed sequencer carries counter_item requests. No custom arbitration is needed for one driver and serially started sequences.

uvm_component_utils enables factory creation. A component constructor takes both name and parent.

001  class counter_sequencer extends uvm_sequencer #(counter_item);

002    `uvm_component_utils(counter_sequencer)

003    function new(string name, uvm_component parent); super.new(name, parent); endfunction

004  endclass

## 9.5  tb/agent/counter_driver.svh

build_phase retrieves the virtual interface. NOVIF is fatal because pin access cannot work without it.

get_next_item blocks until a sequence provides req. At drv_cb the driver maps each request field onto pins. Waiting for mon_cb ensures the active edge has occurred before item_done releases the request handshake.

The next request is driven at the next falling edge. When no request is available, the previous controls stay applied.

001  class counter_driver extends uvm_driver #(counter_item);

002    `uvm_component_utils(counter_driver)

003    virtual counter_if #(WIDTH) vif;

004    function new(string name, uvm_component parent); super.new(name, parent); endfunction

005    function void build_phase(uvm_phase phase);

006      super.build_phase(phase);

007      if (!uvm_config_db#(virtual counter_if #(WIDTH))::get(this, "", "vif", vif))

008        `uvm_fatal("NOVIF", "Driver interface not configured")

009    endfunction

010    task run_phase(uvm_phase phase);

011      forever begin

012        seq_item_port.get_next_item(req);

013        @(vif.drv_cb);

014        vif.drv_cb.valid      <= 1;

015        vif.drv_cb.rst_n      <= req.rst_n;

016        vif.drv_cb.clear      <= req.clear;

017        vif.drv_cb.load       <= req.load;

018        vif.drv_cb.enable     <= req.enable;

019        vif.drv_cb.up         <= req.up;

020        vif.drv_cb.load_value <= req.load_value;

021        @(vif.mon_cb);

022        seq_item_port.item_done();

023      end

024    endtask

025  endclass

## 9.6  tb/agent/counter_monitor.svh

The monitor only observes; it never drives pins and has no sequencer dependency. It is reusable in passive mode.

After each valid sample, it creates a fresh counter_item, copies sampled controls and actual count, then calls ap.write. The two consumers are connected by the environment.

Analysis write is a function and cannot wait; scoreboard and coverage process the sample immediately. Fresh objects prevent later samples overwriting data a subscriber might retain.

001  class counter_monitor extends uvm_monitor;

002    `uvm_component_utils(counter_monitor)

003    virtual counter_if #(WIDTH) vif;

004    uvm_analysis_port #(counter_item) ap;

005    function new(string name, uvm_component parent);

006      super.new(name, parent); ap = new("ap", this);

007    endfunction

008    function void build_phase(uvm_phase phase);

009      super.build_phase(phase);

010      if (!uvm_config_db#(virtual counter_if #(WIDTH))::get(this, "", "vif", vif))

011        `uvm_fatal("NOVIF", "Monitor interface not configured")

012    endfunction

013    task run_phase(uvm_phase phase);

014      counter_item tr;

015      forever begin

016        @(vif.mon_cb);

017        if (vif.mon_cb.valid === 1'b1) begin

018          tr = counter_item::type_id::create("tr");

019          tr.rst_n = vif.mon_cb.rst_n;

020          tr.clear = vif.mon_cb.clear;

021          tr.load = vif.mon_cb.load;

022          tr.enable = vif.mon_cb.enable;

023          tr.up = vif.mon_cb.up;

024          tr.load_value = vif.mon_cb.load_value;

025          tr.count = vif.mon_cb.count;

026          ap.write(tr);

027        end

028      end

029    endtask

030  endclass

## 9.7  tb/agent/counter_agent.svh

The agent groups protocol-specific components. Monitor always exists; driver and sequencer are created only if get_is_active()==UVM_ACTIVE.

connect_phase connects the driver sequence-item port to the sequencer export. Supplied tests assume active mode. A passive integration must supply external stimulus and a different coordinating test.

001  class counter_agent extends uvm_agent;

002    `uvm_component_utils(counter_agent)

003    counter_sequencer sequencer;

004    counter_driver driver;

005    counter_monitor monitor;

006    function new(string name, uvm_component parent); super.new(name, parent); endfunction

007    function void build_phase(uvm_phase phase);

008      super.build_phase(phase);

009      monitor = counter_monitor::type_id::create("monitor", this);

010      if (get_is_active() == UVM_ACTIVE) begin

011        sequencer = counter_sequencer::type_id::create("sequencer", this);

012        driver = counter_driver::type_id::create("driver", this);

013      end

014    endfunction

015    function void connect_phase(uvm_phase phase);

016      super.connect_phase(phase);

017      if (get_is_active() == UVM_ACTIVE)

018        driver.seq_item_port.connect(sequencer.seq_item_export);

019    endfunction

020  endclass

## 9.8  tb/env/counter_scoreboard.svh

uvm_analysis_imp routes monitor writes to this class’s write function. expected is the private reference state, initialized tracks whether it is meaningful.

write applies the independent priority model, compares using !==, increments statistics and prints all useful controls on a mismatch.

check_phase rejects zero compared traffic; report_phase reports totals. UVM_ERROR affects test_passed through the report server.

001  class counter_scoreboard extends uvm_scoreboard;

002    `uvm_component_utils(counter_scoreboard)

003    uvm_analysis_imp #(counter_item, counter_scoreboard) analysis_export;

004    bit [WIDTH-1:0] expected;

005    bit initialized;

006    int unsigned checked, mismatches, up_cycles, down_cycles;

007    integer trace_fd;

008    string trace_path;

009    function void build_phase(uvm_phase phase);

010      super.build_phase(phase);

011      if (!$value$plusargs("COUNTER_CSV=%s", trace_path))

012        trace_path = "sim/build/counter_trace.csv";

013      trace_fd = $fopen(trace_path, "w");

014      if (!trace_fd) `uvm_fatal("TRACE_OPEN", $sformatf("Cannot open counter CSV: %s", trace_path))

015      $fdisplay(trace_fd, "sample,time_ns,operation,rst_n,clear,load,enable,up,load_value,expected,actual,result");

016      `uvm_info("TRACE_FILE", $sformatf("Cycle-by-cycle CSV: %s", trace_path), UVM_LOW)

017    endfunction

018    function new(string name, uvm_component parent);

019      super.new(name, parent); analysis_export = new("analysis_export", this);

020    endfunction

021    function void write(counter_item tr);

022      string operation;

023      if (!tr.rst_n) operation = "RESET";

024      else if (tr.clear) operation = "CLEAR";

025      else if (tr.load) operation = "LOAD";

026      else if (!tr.enable) operation = "HOLD";

027      else if (tr.up) begin operation = "UP"; up_cycles++; end

028      else begin operation = "DOWN"; down_cycles++; end

029      if (!tr.rst_n || tr.clear) begin

030        expected = '0;

031        initialized = 1;

032      end else if (tr.load) begin

033        expected = tr.load_value;

034        initialized = 1;

035      end else if (initialized && tr.enable) begin

036        if (tr.up) expected = expected + 1'b1;

037        else       expected = expected - 1'b1;

038      end

039      if (!initialized) begin

040        `uvm_error("UNINITIALIZED", "First sample must reset, clear, or load the counter")

041        return;

042      end

043      checked++;

044      // Package timescale is 1ns/1ps, so $realtime is expressed in ns here.

045      $fdisplay(trace_fd, "%0d,%0.3f,%s,%b,%b,%b,%b,%b,%0d,%0d,%0d,%s",

046        checked, $realtime, operation, tr.rst_n, tr.clear, tr.load,

047        tr.enable, tr.up, tr.load_value, expected, tr.count,

048        (tr.count === expected) ? "PASS" : "FAIL");

049      $fflush(trace_fd);

050      if (tr.count !== expected) begin

051        mismatches++;

052        `uvm_error("MISMATCH", $sformatf("sample=%0d rst_n=%b clear=%b load=%b en=%b up=%b load_value=0x%0h expected=0x%0h actual=0x%0h",

053          checked, tr.rst_n, tr.clear, tr.load, tr.enable, tr.up, tr.load_value, expected, tr.count))

054      end

055    endfunction

056    function void check_phase(uvm_phase phase);

057      super.check_phase(phase);

058      if (checked == 0) `uvm_error("NO_TRAFFIC", "No samples checked")

059    endfunction

060    function void report_phase(uvm_phase phase);

061      super.report_phase(phase);

062      `uvm_info("SCOREBOARD", $sformatf("Checked %0d samples; mismatches=%0d; UP cycles=%0d; DOWN cycles=%0d", checked, mismatches, up_cycles, down_cycles), UVM_LOW)

063    endfunction

064    function void final_phase(uvm_phase phase);

065      super.final_phase(phase);

066      if (trace_fd) $fclose(trace_fd);

067    endfunction

068  endclass

## 9.9  tb/env/counter_coverage.svh

uvm_subscriber provides an analysis export and a write callback. The covergroup samples explicit arguments, rather than relying on a separate clock.

op categorizes the operation that actually wins priority. previous/have_previous distinguish true wrap opportunities from merely observing a boundary value.

ignore_bins none excludes ordinary cycles from the dedicated wrap-event metric. report_phase prints functional coverage; it does not enforce closure.

001  class counter_coverage extends uvm_subscriber #(counter_item);

002    `uvm_component_utils(counter_coverage)

003    bit [WIDTH-1:0] previous;

004    bit have_previous;

005    // op: reset, clear, load, hold, up, down. Wrap bins use previous count.

006    covergroup cg with function sample(int op, bit direction, bit en,

007                                      bit clr, bit ld, bit reset,

008                                      bit [WIDTH-1:0] value, int wrap);

009      option.per_instance = 1;

010      operation: coverpoint op { bins reset_op={0}; bins clear_op={1};

011        bins load_op={2}; bins hold_op={3}; bins up_op={4}; bins down_op={5}; }

012      count_value: coverpoint value {

013        bins zero={'0}; bins maximum={MAX_COUNT}; bins interior={[1:MAX_COUNT-1]};

014      }

015      wrap_event: coverpoint wrap { bins up_wrap={1}; bins down_wrap={2}; ignore_bins none={0}; }

016      dir_cp: coverpoint direction;

017      en_cp: coverpoint en;

018      clear_cp: coverpoint clr;

019      load_cp: coverpoint ld;

020      reset_cp: coverpoint reset;

021      direction_enable: cross dir_cp, en_cp;

022      clear_load: cross clear_cp, load_cp;

023      reset_load: cross reset_cp, load_cp;

024    endgroup

025    function new(string name, uvm_component parent);

026      super.new(name, parent); cg = new;

027    endfunction

028    function void write(counter_item t);

029      int op, wrap;

030      if (!t.rst_n) op=0;

031      else if (t.clear) op=1;

032      else if (t.load) op=2;

033      else if (!t.enable) op=3;

034      else if (t.up) op=4;

035      else op=5;

036      wrap=0;

037      if (have_previous && op==4 && previous==MAX_COUNT) wrap=1;

038      if (have_previous && op==5 && previous=='0) wrap=2;

039      cg.sample(op, t.up, t.enable, t.clear, t.load, !t.rst_n, t.count, wrap);

040      previous=t.count; have_previous=1;

041    endfunction

042    function void report_phase(uvm_phase phase);

043      super.report_phase(phase);

044      `uvm_info("COVERAGE", $sformatf("Functional coverage: %0.2f%%", cg.get_inst_coverage()), UVM_LOW)

045    endfunction

046  endclass

## 9.10  tb/env/counter_env.svh

build_phase factory-creates agent, scoreboard and coverage. connect_phase broadcasts the monitor analysis port to both subscribers.

The environment knows structure and connections but does not generate test-specific traffic. This lets different tests reuse the same checking infrastructure.

001  class counter_env extends uvm_env;

002    `uvm_component_utils(counter_env)

003    counter_agent agent;

004    counter_scoreboard scoreboard;

005    counter_coverage coverage;

006    function new(string name, uvm_component parent); super.new(name, parent); endfunction

007    function void build_phase(uvm_phase phase);

008      super.build_phase(phase);

009      agent = counter_agent::type_id::create("agent", this);

010      scoreboard = counter_scoreboard::type_id::create("scoreboard", this);

011      coverage = counter_coverage::type_id::create("coverage", this);

012    endfunction

013    function void connect_phase(uvm_phase phase);

014      super.connect_phase(phase);

015      agent.monitor.ap.connect(scoreboard.analysis_export);

016      agent.monitor.ap.connect(coverage.analysis_export);

017    endfunction

018  endclass

## 9.11  tb/sequences/counter_sequences.svh

The base sequence drive helper creates an item, starts the handshake, assigns directed controls and finishes the handshake. Named task arguments make call intent readable.

reset_dut sends two reset cycles and one idle deasserted cycle. Each derived body builds a distinct scenario.

The random sequence reads NUM_ITEMS from a plusarg, calls randomize between start_item and finish_item, and treats constraint failure as UVM_FATAL.

001  class counter_base_seq extends uvm_sequence #(counter_item);

002    `uvm_object_utils(counter_base_seq)

003    function new(string name="counter_base_seq"); super.new(name); endfunction

004    task drive(bit rst_n=1, bit clear=0, bit load=0, bit enable=0,

005               bit up=1, bit [WIDTH-1:0] value='0);

006      counter_item t=counter_item::type_id::create("t");

007      start_item(t);

008      t.rst_n=rst_n; t.clear=clear; t.load=load;

009      t.enable=enable; t.up=up; t.load_value=value;

010      finish_item(t);

011    endtask

012    task reset_dut(); repeat (2) drive(.rst_n(0)); drive(); endtask

013  endclass

014

015  class counter_reset_seq extends counter_base_seq;

016    `uvm_object_utils(counter_reset_seq)

017    function new(string name="counter_reset_seq"); super.new(name); endfunction

018    task body();

019      reset_dut();

020      drive(.load(1), .value(MAX_COUNT));

021      drive(.rst_n(0), .clear(1), .load(1), .enable(1), .value(MAX_COUNT));

022      drive();

023      drive(.load(1), .value(MAX_COUNT));

024      drive(.rst_n(0), .load(1), .enable(1), .value(MAX_COUNT));

025      drive();

026    endtask

027  endclass

028

029  class counter_directed_seq extends counter_base_seq;

030    `uvm_object_utils(counter_directed_seq)

031    function new(string name="counter_directed_seq"); super.new(name); endfunction

032    task body();

033      reset_dut();

034      repeat (12) drive(.enable(1));

035      repeat (6) drive(.enable(1), .up(0));

036      repeat (4) drive(.up(0)); // disabled hold in both directions

037      repeat (4) drive();

038      drive(.load(1), .value(MAX_COUNT >> 1));

039      drive(.load(1), .enable(1), .up(0), .value(MAX_COUNT));

040      drive(.clear(1), .load(1), .enable(1), .value(MAX_COUNT));

041      drive(.clear(1), .enable(1));

042      // Immediate reversals.

043      repeat (8) begin drive(.enable(1)); drive(.enable(1), .up(0)); end

044    endtask

045  endclass

046

047  class counter_wrap_seq extends counter_base_seq;

048    `uvm_object_utils(counter_wrap_seq)

049    function new(string name="counter_wrap_seq"); super.new(name); endfunction

050    task body();

051      reset_dut();

052      drive(.load(1), .value(MAX_COUNT));

053      drive(.enable(1)); // maximum -> zero

054      drive(.enable(1), .up(0)); // zero -> maximum

055      drive(.enable(1));

056      repeat (3) drive(.enable(1), .up(0));

057    endtask

058  endclass

059

060  class counter_random_seq extends counter_base_seq;

061    `uvm_object_utils(counter_random_seq)

062    int unsigned num_items=1000;

063    function new(string name="counter_random_seq"); super.new(name); endfunction

064    task body();

065      counter_item t;

066      reset_dut();

067      void'($value$plusargs("NUM_ITEMS=%d", num_items));

068      repeat (num_items) begin

069        t=counter_item::type_id::create("random_t");

070        start_item(t);

071        if (!t.randomize()) `uvm_fatal("RANDFAIL", "counter_item randomization failed")

072        finish_item(t);

073      end

074    endtask

075  endclass

## 9.12  tb/tests/counter_tests.svh

The base test constructs env and retrieves vif. Its virtual run_sequences method starts reset, directed, wrap and random sequences serially.

run_phase raises an objection, runs the selected sequence(s), drains two monitor clocks, then drops the objection.

The local COUNTER_TEST macro generates four focused test classes overriding run_sequences. It is undefined afterward to avoid leaking a generic macro name. These generated classes are factory-registered.

001  class counter_base_test extends uvm_test;

002    `uvm_component_utils(counter_base_test)

003    counter_env env;

004    virtual counter_if #(WIDTH) vif;

005    function new(string name, uvm_component parent); super.new(name, parent); endfunction

006    function void build_phase(uvm_phase phase);

007      super.build_phase(phase);

008      env=counter_env::type_id::create("env", this);

009      if (!uvm_config_db#(virtual counter_if #(WIDTH))::get(this, "", "vif", vif))

010        `uvm_fatal("NOVIF", "Test interface not configured")

011    endfunction

012    virtual task run_sequences();

013      counter_reset_seq r=counter_reset_seq::type_id::create("reset_seq");

014      counter_directed_seq d=counter_directed_seq::type_id::create("directed_seq");

015      counter_wrap_seq w=counter_wrap_seq::type_id::create("wrap_seq");

016      counter_random_seq s=counter_random_seq::type_id::create("random_seq");

017      r.start(env.agent.sequencer);

018      d.start(env.agent.sequencer);

019      w.start(env.agent.sequencer);

020      s.start(env.agent.sequencer);

021    endtask

022    task run_phase(uvm_phase phase);

023      phase.raise_objection(this);

024      run_sequences();

025      // Drain monitor/analysis callbacks for the last driven transaction.

026      repeat (2) @(vif.mon_cb);

027      phase.drop_objection(this);

028    endtask

029  endclass

030

031  `define COUNTER_TEST(CLASS, SEQ) class CLASS extends counter_base_test;   `uvm_component_utils(CLASS)   function new(string name, uvm_component parent); super.new(name,parent); endfunction   virtual task run_sequences();     SEQ s=SEQ::type_id::create("seq");     s.start(env.agent.sequencer);   endtask endclass

032

033  `COUNTER_TEST(counter_reset_test, counter_reset_seq)

034  `COUNTER_TEST(counter_directed_test, counter_directed_seq)

035  `COUNTER_TEST(counter_wrap_test, counter_wrap_seq)

036  `COUNTER_TEST(counter_random_test, counter_random_seq)

037  `undef COUNTER_TEST

## 9.13  tb/counter_pkg.sv

The package defines WIDTH from the compile-time COUNTER_WIDTH macro and defines MAX_COUNT as a WIDTH-bit all-ones value.

uvm_pkg supplies UVM types and uvm_macros.svh supplies registration/reporting macros. Include order matters: items before typed components, agent children before agent, environment children before env, sequences before tests.

The .svh files are class fragments included inside the package; compile counter_pkg.sv once, not each header as a separate compilation unit.

001  `timescale 1ns/1ps

002  `ifndef COUNTER_WIDTH

003    `define COUNTER_WIDTH 8

004  `endif

005  package counter_pkg;

006    import uvm_pkg::*;

007    `include "uvm_macros.svh"

008    parameter int WIDTH = `COUNTER_WIDTH;

009    parameter bit [WIDTH-1:0] MAX_COUNT = {WIDTH{1'b1}};

010    `include "agent/counter_item.svh"

011    `include "agent/counter_sequencer.svh"

012    `include "agent/counter_driver.svh"

013    `include "agent/counter_monitor.svh"

014    `include "agent/counter_agent.svh"

015    `include "env/counter_scoreboard.svh"

016    `include "env/counter_coverage.svh"

017    `include "env/counter_env.svh"

018    `include "sequences/counter_sequences.svh"

019    `include "tests/counter_tests.svh"

020  endpackage

## 9.14  tb/top/tb_top.sv

tb_top elaborates the DUT and interface and generates the clock. Port connections tie interface signals directly to DUT pins.

The config DB set makes the same virtual interface available to uvm_test_top and descendants. run_test chooses the class from UVM_TESTNAME.

finish_on_completion=0 returns control after UVM reporting. The report server counts determine test_passed before $finish; a separate watchdog prevents endless simulation.

001  `timescale 1ns/1ps

002  module tb_top;

003    import uvm_pkg::*;

004    import counter_pkg::*;

005    bit clk=0;

006    bit test_passed=0;

007    always #5 clk=~clk;

008    counter_if #(WIDTH) vif(clk);

009    up_down_counter #(.WIDTH(WIDTH)) dut (

010      .clk(clk), .rst_n(vif.rst_n), .clear(vif.clear), .load(vif.load),

011      .enable(vif.enable), .up(vif.up), .load_value(vif.load_value), .count(vif.count)

012    );

013    initial begin

014      if (WIDTH < 2) $fatal(1, "UVM environment requires WIDTH >= 2");

015      uvm_config_db#(virtual counter_if #(WIDTH))::set(null, "uvm_test_top*", "vif", vif);

016      // Retain control after UVM report phase so CI can inspect an explicit result.

017      uvm_top.finish_on_completion = 0;

018      run_test();

019      begin

020        uvm_report_server reports;

021        reports = uvm_report_server::get_server();

022        test_passed = (reports.get_severity_count(UVM_ERROR) == 0 &&

023                       reports.get_severity_count(UVM_FATAL) == 0);

024      end

025      $finish;

026    end

027    // Independent watchdog prevents a hung sequencer/objection from running forever.

028    initial begin

029      #10ms;

030      $fatal(1, "Simulation watchdog expired");

031    end

032  endmodule

## 9.15  tb/rtl_smoke/counter_rtl_smoke.sv

This is an alternative non-UVM test top, not part of the UVM hierarchy. The step task drives on a falling edge, predicts state, waits for a rising edge and checks after a small delay.

The directed section checks priority and wrap. A reset asserted between active edges is checked before the next rising edge. The random loop adds 2,000 comparisons.

Use make smoke with Icarus on a suitable system. The Windows UVM scripts intentionally do not compile this top.

001  `timescale 1ns/1ps

002  // Portable RTL-only test. This supplements, not replaces, the UVM regression.

003  module counter_rtl_smoke;

004    parameter integer WIDTH=8;

005    reg clk=0;

006    always #5 clk=~clk;

007    reg rst_n=1, clear=0, load=0, enable=0, up=1;

008    reg [WIDTH-1:0] load_value=0;

009    wire [WIDTH-1:0] count;

010    reg [WIDTH-1:0] expected=0;

011    reg [WIDTH-1:0] max_count={WIDTH{1'b1}};

012    integer checked=0;

013    integer i;

014    up_down_counter #(.WIDTH(WIDTH)) dut(.*);

015

016    task step(input reg r, c, l, e, u, input reg [WIDTH-1:0] v);

017      begin

018        @(negedge clk);

019        rst_n=r; clear=c; load=l; enable=e; up=u; load_value=v;

020        if (!r || c) expected=0;

021        else if (l) expected=v;

022        else if (e && u) expected=expected+1'b1;

023        else if (e) expected=expected-1'b1;

024        @(posedge clk); #1;

025        checked=checked+1;

026        if (count !== expected)

027          $fatal(1,"sample %0d: expected %h got %h",checked,expected,count);

028      end

029    endtask

030

031    initial begin

032      if (WIDTH < 1) $fatal(1,"WIDTH must be positive");

033      step(0,0,0,0,1,0);

034      step(1,0,1,1,1,max_count);

035      step(1,0,0,1,1,0); // overflow

036      step(1,0,0,1,0,0); // underflow

037      step(1,0,0,0,0,0); // hold

038      step(1,1,1,1,1,max_count); // clear wins over load and enable

039      step(1,0,1,0,0,max_count); // load works with enable low

040      // Assert reset BETWEEN active clock edges and check before the next edge.

041      #1; rst_n=0; #1;

042      if (count !== 0) $fatal(1,"Asynchronous reset failed before clock edge");

043      expected=0;

044      step(0,1,1,1,0,max_count); // reset wins over all controls

045      step(1,0,0,0,1,0);

046      for (i=0;i<2000;i=i+1)

047        step(($urandom_range(0,49)!=0), ($urandom_range(0,19)==0),

048             ($urandom_range(0,9)==0), $urandom_range(0,1),

049             $urandom_range(0,1), $urandom);

050      $display("RTL SMOKE PASS: WIDTH=%0d, %0d checked cycles", WIDTH, checked);

051      $finish;

052    end

053    initial begin #1ms; $fatal(1,"Smoke watchdog expired"); end

054  endmodule

## 9.16  run.do

This is the Windows-friendly root entry. It resolves its own folder, sets defaults, reads UVM_HOME from Tcl or environment, and tries the typical simulator source location if unset.

It validates basic configuration, builds a Tcl argument list so paths with spaces remain intact, compiles in dependency order and loads the selected test.

GUI_MODE=1 adds waves and keeps the session open. Batch mode delegates completion and exit status to sim/run.do. catch handles compile/load errors.

001  # Windows/Linux native Questa flow. No make or shell is required.

002  # GUI Transcript: set UVM_HOME {C:/tools/uvm-1.2}; set GUI_MODE 1; do run.do

003  # Batch CMD:     set "UVM_HOME=C:\tools\uvm-1.2"  then run.bat

004  # Optional Tcl variables: TEST, WIDTH, SEED, NUM_ITEMS, GUI_MODE.

005  set PROJECT_ROOT [file normalize [file dirname [info script]]]

006  cd $PROJECT_ROOT

007  foreach {name default} {TEST counter_base_test WIDTH 8 SEED 1 NUM_ITEMS 1000 GUI_MODE 0} {

008      if {![info exists $name]} {set $name $default}

009  }

010  if {![info exists UVM_HOME] && [info exists env(UVM_HOME)]} {

011      set UVM_HOME $env(UVM_HOME)

012  }

013  if {![info exists UVM_HOME]} {

014      # Typical Questa installation: win64/vsim.exe beside verilog_src/uvm-1.2.

015      set candidate [file normalize [file join [file dirname [info nameofexecutable]] .. verilog_src uvm-1.2]]

016      if {[file exists [file join $candidate src uvm_pkg.sv]]} {set UVM_HOME $candidate}

017  }

018  if {![info exists UVM_HOME] || ![file exists [file join $UVM_HOME src uvm_pkg.sv]]} {

019      puts stderr "ERROR: Set UVM_HOME to the UVM source folder containing src/uvm_pkg.sv."

020      puts stderr {Example: set UVM_HOME {C:/questasim64_2024.3/verilog_src/uvm-1.2}}

021      if {!$GUI_MODE} {quit -code 1 -force}

022      return

023  }

024  if {![string is integer -strict $WIDTH] || $WIDTH < 2 ||

025      ![string is integer -strict $NUM_ITEMS] || $NUM_ITEMS < 1} {

026      puts stderr "ERROR: WIDTH must be >= 2; NUM_ITEMS must be >= 1."

027      if {!$GUI_MODE} {quit -code 1 -force}

028      return

029  }

030  set BUILD [file join $PROJECT_ROOT sim build]

031  set WORK [file join $BUILD work]

032  file mkdir $BUILD

033  set RUN_NAME ${TEST}_seed${SEED}

034  set TRACE [file join $BUILD ${RUN_NAME}_counter.csv]

035  set WAVEFILE [file join $BUILD ${RUN_NAME}.wlf]

036  # Explicit transcript selection works when this script is sourced in an open GUI.

037  transcript file [file join $BUILD ${RUN_NAME}.log]

038  transcript on

039  puts "Selected test=$TEST width=$WIDTH seed=$SEED random_items=$NUM_ITEMS"

040  puts "Run files: [file join $BUILD $RUN_NAME].log / .wlf / _counter.csv"

041  if {[catch {

042      if {![file isdirectory $WORK]} {vlib $WORK}

043      set args [list -sv -timescale 1ns/1ps -work $WORK +define+UVM_NO_DPI \

044          +define+COUNTER_WIDTH=$WIDTH "+incdir+[file join $PROJECT_ROOT tb]" \

045          "+incdir+[file join $UVM_HOME src]" [file join $UVM_HOME src uvm_pkg.sv]]

046      foreach source {rtl/up_down_counter.sv tb/top/counter_if.sv tb/counter_pkg.sv tb/top/tb_top.sv} {

047          lappend args [file join $PROJECT_ROOT $source]

048      }

049      vlog {*}$args

050      vsim -voptargs=+acc -lib $WORK tb_top -sv_seed $SEED \

051          +UVM_TESTNAME=$TEST +NUM_ITEMS=$NUM_ITEMS \

052          "+COUNTER_CSV=$TRACE" -wlf $WAVEFILE

053  } message]} {

054      puts stderr "BUILD/LOAD FAILED: $message"

055      if {!$GUI_MODE} {quit -code 1 -force}

056      return

057  }

058  # Record from time zero, including batch mode, for later waveform debugging.

059  log -r /tb_top/vif/*

060  if {$GUI_MODE} {

061      onfinish stop

062      add wave -radix unsigned /tb_top/vif/count /tb_top/vif/load_value

063      add wave /tb_top/vif/clk /tb_top/vif/rst_n /tb_top/vif/clear /tb_top/vif/load /tb_top/vif/enable /tb_top/vif/up

064      run -all

065      wave zoom full

066      puts "Simulation stopped. Review the UVM summary and /tb_top/test_passed (1 means PASS)."

067  } else {

068      do [file join $PROJECT_ROOT sim run.do]

069  }

## 9.17  sim/run.do

This batch helper converts simulator errors/breaks to failure exit codes. onfinish stop lets Tcl continue after the test calls $finish.

After run -all returns, it examines test_passed and quits with status 0 only for an explicit success. It must run after tb_top has been loaded.

001  # Invoked from up_down_counter/ by Makefile after loading tb_top.

002  # A watchdog, assertion, elaboration failure, or UVM error must fail automation.

003  onerror {quit -code 1 -force}

004  onbreak {quit -code 1 -force}

005  onfinish stop

006  log -r /tb_top/vif/*

007  run -all

008  if {[examine -radix unsigned /tb_top/test_passed] != 1} {

009      quit -code 1 -force

010  }

011  quit -code 0 -force

## 9.18  run.bat

The Windows launcher changes to its own directory and sets positional-argument defaults. where vsim diagnoses a missing PATH entry.

It starts Questa in command-line mode, passes configuration as Tcl variables and returns the simulator’s exit code. Use Command Prompt for the examples.

001  @echo off

002  setlocal

003  rem Run from Windows Command Prompt. Add Questa's win64 directory to PATH.

004  rem Usage: run.bat [test_name] [seed] [width] [num_items]

005  cd /d "%~dp0"

006  set "TEST=counter_base_test"

007  set "SEED=1"

008  set "WIDTH=8"

009  set "NUM_ITEMS=1000"

010  if not "%~1"=="" set "TEST=%~1"

011  if not "%~2"=="" set "SEED=%~2"

012  if not "%~3"=="" set "WIDTH=%~3"

013  if not "%~4"=="" set "NUM_ITEMS=%~4"

014  where vsim >nul 2>nul

015  if errorlevel 1 (

016    echo ERROR: vsim is not on PATH. Add the Questa win64 directory to PATH.

017    exit /b 1

018  )

019  vsim -c -do "set TEST {%TEST%}; set SEED {%SEED%}; set WIDTH {%WIDTH%}; set NUM_ITEMS {%NUM_ITEMS%}; do run.do"

020  exit /b %ERRORLEVEL%

## 9.19  regression.bat

The loop calls run.bat once for each test. call is necessary so execution returns to this batch file after the nested batch completes.

The first failed test exits with code 1; only completing the loop prints REGRESSION PASS.

001  @echo off

002  setlocal

003  cd /d "%~dp0"

004  for %%T in (counter_reset_test counter_directed_test counter_wrap_test counter_random_test counter_base_test) do (

005    call run.bat %%T 1 8 1000

006    if errorlevel 1 exit /b 1

007  )

008  echo REGRESSION PASS

009  exit /b 0

## 9.20  Makefile

This optional GNU make flow is for a bash-based environment, not required on Windows. It compiles local UVM sources with UVM_NO_DPI and the counter width define.

run and regression use the batch Tcl helper; smoke invokes Icarus on the separate RTL top. clean removes only sim/build.

001  SHELL := /bin/bash

002  .DEFAULT_GOAL := help

003  VLIB ?= vlib

004  VLOG ?= vlog

005  VSIM ?= vsim

006  IVERILOG ?= iverilog

007  VVP ?= vvp

008  WIDTH ?= 8

009  TEST ?= counter_base_test

010  SEED ?= 1

011  NUM_ITEMS ?= 1000

012  # UVM source is compiled locally, avoiding vendor-specific library mappings/DPI.

013  # Set UVM_HOME explicitly, or set QUESTA_HOME to the simulator install root.

014  UVM_HOME ?= $(if $(QUESTA_HOME),$(QUESTA_HOME)/verilog_src/uvm-1.2,)

015  BUILD := sim/build

016  SOURCES := rtl/up_down_counter.sv tb/top/counter_if.sv tb/counter_pkg.sv tb/top/tb_top.sv

017  TESTS := counter_reset_test counter_directed_test counter_wrap_test counter_random_test counter_base_test

018  .PHONY: help compile run regression smoke clean

019  help:

020  	@echo 'make run [TEST=counter_base_test SEED=1 WIDTH=8 NUM_ITEMS=1000]'

021  	@echo 'make regression   # five UVM tests, independent logs and failure exit codes'

022  	@echo 'make smoke        # standalone RTL test using Icarus; no UVM required'

023  	@echo 'make clean        # remove only generated simulation build files'

024  compile:

025  	@test -n "$(UVM_HOME)" -a -f "$(UVM_HOME)/src/uvm_pkg.sv" || { echo 'Set UVM_HOME to a UVM 1.2 (or IEEE UVM) source directory containing src/uvm_pkg.sv'; exit 1; }

026  	@mkdir -p $(BUILD)

027  	@test -d $(BUILD)/work || $(VLIB) $(BUILD)/work

028  	$(VLOG) -sv -timescale 1ns/1ps -work $(BUILD)/work +define+UVM_NO_DPI +define+COUNTER_WIDTH=$(WIDTH) +incdir+tb +incdir+$(UVM_HOME)/src $(UVM_HOME)/src/uvm_pkg.sv $(SOURCES)

029  run: compile

030  	$(VSIM) -c -lib $(BUILD)/work tb_top -sv_seed $(SEED) +UVM_TESTNAME=$(TEST) +NUM_ITEMS=$(NUM_ITEMS) +COUNTER_CSV=$(BUILD)/$(TEST)_seed$(SEED)_counter.csv -wlf $(BUILD)/$(TEST)_seed$(SEED).wlf -l $(BUILD)/$(TEST)_seed$(SEED).log -do sim/run.do

031  regression: compile

032  	@set -e; for test in $(TESTS); do \

033  	  $(VSIM) -c -lib $(BUILD)/work tb_top -sv_seed $(SEED) \

034  	    +UVM_TESTNAME=$$test +NUM_ITEMS=$(NUM_ITEMS) +COUNTER_CSV=$(BUILD)/$${test}_seed$(SEED)_counter.csv \

035  	    -wlf $(BUILD)/$${test}_seed$(SEED).wlf \

036  	    -l $(BUILD)/$${test}_seed$(SEED).log -do sim/run.do; \

037  	done

038  smoke:

039  	@mkdir -p $(BUILD)

040  	$(IVERILOG) -g2012 -s counter_rtl_smoke -Pcounter_rtl_smoke.WIDTH=$(WIDTH) -o $(BUILD)/smoke.vvp rtl/up_down_counter.sv tb/rtl_smoke/counter_rtl_smoke.sv

041  	$(VVP) $(BUILD)/smoke.vvp

042  clean:

043  	rm -rf $(BUILD)

# 10. Reading both directions and the new counter logs

For a clearly visible ramp, run counter_directed_test with WIDTH=8. After reset it has twelve UP cycles (1 through 12) followed by six DOWN cycles (11 through 6). counter_wrap_test has only isolated increments at the modulo boundary and ends with decrement traffic; it is not intended as a long ramp demo.

The updated root run.do explicitly selects and enables the transcript, names the waveform database, records interface signals from time zero and passes a per-run CSV path. The scoreboard writes and flushes a CSV row per checked cycle, closes it in final_phase and reports UP and DOWN cycle totals.

Outputs for a directed test with seed 1 are sim/build/counter_directed_test_seed1.log, sim/build/counter_directed_test_seed1.wlf, and sim/build/counter_directed_test_seed1_counter.csv. Open CSV in Excel and filter operation to UP or DOWN.

CSV columns: sample, time_ns, operation, rst_n, clear, load, enable, up, load_value, expected, actual and result. Effective operation is RESET, CLEAR, LOAD, HOLD, UP or DOWN according to control priority. Actual count retains X/Z, which fails comparison.

The original package had UVM summary logging, not a per-cycle counter trace. These CSV and transcript/WLF improvements are new. Repeating a test/seed replaces the CSV/WLF; copy previous outputs before rerunning if needed. Run the root script rather than manual run -all and check write permissions if files are missing.

The updated scoreboard statically compiles and elaborates against UVM 1.2 at widths 2, 8 and 16. Windows Questa execution and runtime artifact generation remain unverified in the sandbox. No actual simulation trace or coverage percentage is claimed.

# 11. Validation record and next steps

pyslang 12.1.0 static compilation/elaboration against unmodified Accellera UVM 1.2 passed at WIDTH=2, 8 and 16. The two warnings originated in upstream UVM empty-loop bodies.

A static check against IEEE UVM reference sources also passed, with one upstream escape-sequence warning.

The RTL-only smoke top passed static compilation without warnings. Windows Tcl and batch launchers were reviewed, but not executed in a Windows Questa installation.

No HDL simulation was executed in the development workspace because Questa/ModelSim and Icarus are unavailable. Runtime pass, Windows compatibility and coverage numbers must be confirmed on your licensed local installation.

Recommended local acceptance: run counter_wrap_test in GUI and inspect both boundaries; run regression.bat; then run constrained-random tests with several seeds and retain logs. Report any first failure with the test name, seed, width, item count, simulator version and the first error message.